# Technical Optimization of High-Frame-Rate Data Acquisition for Smartphone-Based Particle Image Velocimetry on the Samsung Galaxy S25

# Engineering Foundations of Smartphone-Based Particle Image Velocimetry

The discipline of experimental fluid mechanics has long relied upon Particle Image Velocimetry (PIV) to map complex velocity fields by tracking the displacement of tracer particles within a fluid medium. Traditionally, this required laboratory-grade hardware, including high-speed CMOS cameras capable of thousands of frames per second, high-energy pulsed lasers, and specialized frame grabbers. However, the rapid advancement of mobile imaging hardware, exemplified by the Samsung Galaxy S25 series, has democratized high-frame-rate (HFR) acquisition. The objective of a Smartphone-Based PIV (S-PIV) Data Acquisition tool is to leverage these consumer devices to perform scientific measurements, which necessitates a shift from perceptual image quality—the

primary goal of consumer photography—to raw data integrity.

The central challenge in S-PIV is the acquisition of frames at a temporal resolution sufficient to resolve small-scale turbulent structures. For many aqueous or air-flow applications, 240 frames per second (fps) represents the minimum threshold for viable data. On the Samsung Galaxy S25, achieving a sustained 240 fps capture session is a non-trivial engineering task that involves bypassing the standard Android media framework, interacting directly with the Camera2 API's constrained high-speed capture sessions, and managing the massive data throughput generated by uncompressed luminance data. This report analyzes the mobile camera data, the project specifications for the HighFPSRecorder_240fps repository, and the technical strategies required to elevate the primary camera's capture capability from standard 60 fps to the hardware-limit of 240

fps.

# Hardware and Sensor Architecture of the Samsung Galaxy S25

## Primary Sensor Characteristics: ISOCELL GN3 and HP2

The Samsung Galaxy S25 ecosystem utilizes two distinct sensor architectures for its primary wide- angle cameras depending on the model tier. The base S25 and S25+ models utilize the 50

megapixel (MP) ISOCELL GN3 sensor, while the S25 Ultra features the 200 MP ISOCELL HP2 sensor. For PIV applications, the sensor's physical size and pixel pitch are paramount, as they dictate the photon collection efficiency and the diffraction limits of the optical system.

| Feature | Samsung Galaxy S25/S25+ | Samsung Galaxy S25 Ultra |
| --- | --- | --- |
|  | (GN3) | (HP2) |
| Max Resolution | 50 MP (8160 x 6120) | 200 MP (16320 x 12240) |
| Sensor Format | 1/1.56" | 1/1.3" |
| Individual Pixel Size | 1.0 μm | 0.6 μm |

| Feature | Samsung Galaxy S25/S25+ (GN3) | Samsung Galaxy S25 Ultra (HP2) |
| --- | --- | --- |
| Binned Pixel Size | 2.0 μm (12.5 MP) | 2.4 μm (12.5 MP) |
| Focusing System | Dual Pixel PDAF | Multi-directional PDAF |
| High Speed Support | 1080p @ 240 fps | 1080p @ 240 fps / 4K @ 120 fps |

The GN3 sensor in the standard S25 is highly suitable for PIV because its 1.0 μm native pixels provide a better signal-to-noise ratio (SNR) than the 0.6 μm pixels of the HP2 when both are capturing in unbinned modes. In PIV, particles often occupy only 2-3 pixels in diameter; thus, preserving high contrast and minimal noise at the pixel level is critical for the sub-pixel accuracy of cross-correlation algorithms. The GN3 employs Front Deep Trench Isolation (FDTI) to reduce

crosstalk, which is essential when imaging bright laser-illuminated particles against a dark background.

## Image Signal Processing and the Snapdragon 8 Elite

C	C

The S25 is powered by the Qualcomm Snapdragon 8 Elite chipset, which includes a sophisticated Image Signal Processor (ISP) capable of processing data at gigapixel-per-second rates. For standard video, the ISP performs real-time noise reduction, sharpening, and color grading through the ProVisual Engine. In a PIV context, these enhancements are detrimental. AI-based noise reduction often interprets small tracer particles as sensor noise and smooths them out, while

sharpening creates ringing artifacts that distort particle centroids.

To achieve 240 fps, the ISP must operate in a specialized "constrained" mode where many of its high-level processing blocks are bypassed or simplified to ensure the sensor readout can match the requested frequency. This provides the scientific advantage of obtaining data that is much closer to the raw sensor output than standard video files.

# Software Interface: The Camera2 API and High-Speed Capture Sessions

## Mechanisms of createConstrainedHighSpeedCaptureSession

The Camera2 API, introduced in Android 5.0, provides the low-level hooks necessary for manual sensor control. For high-frame-rate applications exceeding 60 fps, the standard

CameraCaptureSession is insufficient. Developers must use

createConstrainedHighSpeedCaptureSession , a specialized interface designed for high-throughput video streams.

In a constrained high-speed session, the Android Hardware Abstraction Layer (HAL) takes over many of the scheduling tasks typically handled by the application to minimize the overhead of the

Java-to-Native transition. The session enforces strict constraints: it only accepts a maximum of two output surfaces (typically a preview surface and a recording/analysis surface) and requires all capture requests to be submitted as a batched list via createHighSpeedRequestList . This batching allows the ISP to aggregate multiple frames into single processing units, which is necessary to maintain the 240 fps cadence on the S25 hardware.

## The Disparity Between Logical and Physical Cameras

The most significant hurdle identified in the developer's project is that the primary camera (ID 0) defaults to a 60 fps cap, while only the "slow-motion" mode (likely tied to a specific physical sensor or Mode) reaches 240 fps. On Samsung devices, "Camera 0" is often a "Logical Camera"—a software wrapper that manages multiple physical sensors.

The logical camera is tuned for stability. When a third-party app requests a high-speed range, the logical camera may reject it because it cannot guarantee consistent performance across all its underlying physical lenses (wide, ultra-wide, telephoto). To overcome the 60 fps limitation, the application must identify the specific Physical Camera ID that corresponds to the main wide-angle sensor. Research into Samsung's HAL mapping reveals that while the logical ID is 0, the actual primary sensor supporting full high-speed RAW access is frequently mapped to a hidden ID, such

as ID 56 on newer flagship iterations.

Camera ID

Type	Capability	PIV Suitability

ID 0	Logical Main	Combined wide/tele; 60 fps cap for 3rd

party apps.

Low (Rate capped)

ID 1	Physical Front	12 MP; typically limited to 60 fps.	Poor

ID 2	Physical Ultra- Wide

12/50 MP; supports 240 fps but has heavy distortion.

Moderate

ID 56	Physical Primary

The raw GN3/HP2 sensor; can often unlock 240 fps RAW.

High (Goal)

C	C

By querying CameraCharacteristics.getPhysicalCameraIds() and inspecting the

StreamConfigurationMap for each physical ID, the developer can locate the sensor that explicitly lists 240 fps in its getHighSpeedVideoFpsRangesFor output.

# Manual Exposure Control for High-Speed Acquisition

## Temporal Requirements in PIV

PIV relies on the displacement of particles over a known time interval, Δt. At 240 fps, the time between consecutive frames is constant at Δt = 1/240 ≈ 4.167 ms. However, the exposure time ( texp) of each individual frame must be significantly shorter than the frame duration to prevent

"streaking" of the particles. If a particle moves 10 pixels during the exposure, it will appear as a blur

rather than a point, leading to high errors in correlation.

For most S-PIV setups using continuous wave (CW) lasers, an exposure time of 500μs to 1ms is ideal. The Samsung Galaxy S25 provides the necessary manual controls through the

CaptureRequest keys SENSOR_EXPOSURE_TIME and SENSOR_SENSITIVITY .

## Implementing Manual Controls via Camera2

To implement these controls within a high-speed session on the S25:

- Disable Auto-Exposure: The key CONTROL_AE_MODE must be set to OFF . If left on ON , the system will attempt to optimize for "brightness," often resulting in a frame rate drop in low-light PIV environments to increase exposure time.

- Define Exposure Duration: The SENSOR_EXPOSURE_TIME is set in nanoseconds. For a 1 ms exposure, the value is 1,000,000. The user's repo should allow for a slider or direct input for this value to match the flow velocity.

- Optimize ISO (Sensitivity): Since PIV illumination is often limited, the SENSOR_SENSITIVITY must be increased to capture the faint scattering from particles. The ISOCELL GN3 sensor performs well up to ISO 1600 or 3200, beyond which thermal noise begins to obscure tracer particles.

- Lock White Balance: Set CONTROL_AWB_MODE to OFF and use a manual gain to ensure that color-correction algorithms do not shift the luminance values of the Y-plane between frames.

# Y-Plane Extraction and Data Management

## The Importance of Raw Luminance

PIV cross-correlation is fundamentally a grayscale operation. Converting sensor data to RGB (Red, Green, Blue) and then to grayscale in post-processing is computationally wasteful and introduces interpolation artifacts known as "demosaicing" noise. The objective is to extract the Y-plane (Luminance) directly from the YUV_420_888 format, which is the standard format for analysis-ready

camera data on Android.

In a YUV_420_888 image, the data is partitioned into three planes:

Plane 0 (Y): Contains the brightness information for every pixel.

Planes 1 & 2 (U and V): Contain chrominance information at half-resolution (subsampled).

By focusing exclusively on Plane 0, the PIV tool can reduce the incoming data rate by one-third, allowing the S25's processor to keep pace with the 240 fps stream.

## Navigating Stride and Memory Alignment

A common error in high-speed Y-plane extraction is the failure to account for "Row Stride." On the S25, the internal hardware requires that each row of image data be aligned to a specific memory boundary (e.g., 1024 bytes). If a developer captures a 1080p frame (1920 pixels wide), the

rowStride might actually be 2048 bytes. The "extra" bytes are padding used for memory efficiency by the ISP.

If the application attempts to read the Y-plane as a single block of 1920 * 1080 bytes, the resulting images will be skewed. The correct extraction logic involves iterating through each row and copying exactly width bytes, then skipping the padding before reading the next row. This must be implemented in the Android NDK (Native Development Kit) to ensure it executes within the ≈ 4ms

window available between frames at 240 fps.

# Circular Buffer Implementation for Continuous Capture

## Handling High-Speed Throughput

At 240 fps and 1080p resolution, the Y-plane alone generates a data stream of:

1920 × 1080 pixels × 1 byte/pixel × 240 fps ≈ 497.66 MB/s

Most smartphone storage systems (including the S25's UFS 4.0) struggle to sustain this write speed for long periods without causing the camera buffer to overflow. To solve this, a circular buffer is implemented in the device's RAM.

A circular buffer is a fixed-size memory structure where new data overwrites the oldest data in a continuous loop. For PIV, the researcher typically observes the flow and triggers a "save" after a specific event occurs. The circular buffer ensures that the system is always "recording" the last few seconds of motion in high fidelity, without the latency associated with starting a recording session.

## Circular Buffer Architecture and NDK Integration

The circular buffer should be managed in native C++ to prevent the Android Garbage Collector (GC) from interrupting the HFR stream. The architecture consists of:

Pre-allocated Memory: A block of RAM (e.g., 2 GB) is allocated at startup, capable of holding approximately 4 seconds of 240 fps Y-plane data (2048 MB/497 MB/s ≈ 4.1 seconds).

Queue Management: The onImageAvailable callback from the ImageReader retrieves the latest frame and immediately passes the pointer to the NDK buffer.

Atomic Indexing: Thread-safe counters manage the "write" and "read" heads of the buffer to ensure that frames are not corrupted during the "dump-to-disk" phase.

Asynchronous Disk I/O: When the user triggers a capture, a background thread writes the contents of the buffer to UFS 4.0 storage in large chunks to maximize write throughput.

# PIV Project Specification: Smartphone-Based Acquisition Tool

The following specifications define the functional requirements for a robust S-PIV acquisition tool on the Samsung Galaxy S25 platform, addressing the current limitations found in the developer's 60 fps primary camera implementation.

## Technical Requirements Table

Requirement	Specification	Implementation Strategy

Temporal Frequency

Sustained 240 Hz

Use CameraConstrainedHighSpeedCaptureSession

with Physical ID 56.

Requirement	Specification	Implementation Strategy

Spatial Resolution

1920 x 1080 (FHD)

Target resolution for high-speed sensor readout.

Data Format	Raw Y-Plane

(Luminance)

Extract Image.Plane from YUV_420_888 .

Exposure Control

Manual 100 μs - 4 ms

Set CONTROL_AE_MODE_OFF and

SENSOR_EXPOSURE_TIME .

Sensitivity	ISO 50 - 3200	Set SENSOR_SENSITIVITY manually to control SNR.

Buffering	RAM-resident Circular Buffer

2 GB pre-allocated memory pool in NDK.

Timestamps	Nanosecond

Precision

Record CaptureResult.SENSOR_TIMESTAMP for every frame.

## Functional Logic and Session Lifecycle

C	C

Device Enumeration: The tool must iterate through all physical camera IDs. It should not stop at the logical "Camera 0" but search for the physical sensor (likely ID 56 or similar) that supports CONSTRAINED_HIGH_SPEED_VIDEO .

- Session Creation: Initialize a SessionConfiguration.SESSION_HIGH_SPEED . The output configurations must include an ImageReader configured with ImageFormat.YUV_420_888 and a maxImages count of at least 8 to prevent frame dropping during momentary CPU spikes.

- High-Speed Request Batching: Construct a CaptureRequest with manual exposure and fixed 240 fps range. Use createHighSpeedRequestList to generate the sequence of requests required by the HAL.

- Real-Time Processing Pipeline: The ImageReader.OnImageAvailableListener must execute minimal code. It should hand off the buffer to a native NDK thread that handles the Y-plane copy and stride correction.

- Visualization Thread: Simultaneously, a lower-priority thread should downsample the Y- plane data and render it to a SurfaceView using OpenGL ES. This provides the user with a

30 or 60 fps preview of the 240 fps acquisition for alignment purposes.

# Bypassing System-Level Constraints and Optimizations

## Solving the 60 fps Primary Camera Bottleneck

The developer's observation that 240 fps is only enabled for the "slow-motion" camera suggests that the Samsung HAL is reporting limited high-speed ranges for the logical ID 0. On the S25, this is often a software enforcement to manage heat. To bypass this and achieve 240 fps on the primary GN3 sensor:

Direct Sensor Addressing: Instead of cameraManager.openCamera("0",...) , the app must open the specific physical ID associated with the 50 MP sensor.

Template Targeting: Use CameraDevice.TEMPLATE_RECORD when creating the capture request. This template signals to the ISP that high-speed throughput is required, potentially enabling internal hardware paths that TEMPLATE_PREVIEW does not.

Resolution Matching: High-speed modes on the GN3 sensor are often limited to specific resolutions where the sensor can perform 2x2 or 4x4 binning. The app must request exactly 1920x1080 or 1280x720, as arbitrary resolutions will default to the standard 60 fps pipeline.

## Disabling Computational Photography

To ensure the integrity of the PIV data, every "intelligent" feature of the S25 must be suppressed.

Intelligent Optimization: Set to "Minimum" in the system settings or bypass via API.

Scene Optimizer: Set CONTROL_SCENE_MODE to DISABLED .

Video Stabilization: CONTROL_VIDEO_STABILIZATION_MODE must be OFF , as electronic image stabilization (EIS) crops and warps the frame to compensate for motion, which would invalidate the PIV velocity vectors.

HDR and Tone Mapping: Set TONEMAP_MODE to CONTRAST_CURVE or RAW to prevent the ISP from altering the local contrast of the tracer particles.

# Comparative Analysis of High-Speed Acquisition Platforms

The Samsung Galaxy S25 occupies a unique position in the S-PIV landscape when compared to its primary competitors, the iPhone 16 Pro and the Google Pixel 9 Pro.

Feature	Samsung Galaxy S25	iPhone 16 Pro	Google Pixel 9 Pro

API Openness	Moderate (Complex

mapping)

Low (Restricted)	High (Standard Camera2)

HFR RAW

Support

Possible via Physical IDs.

Highly restricted.	Supported via Standard API.

Sensor (Main)	50 MP GN3 (1.0 μm)	48 MP (1.22 μm)	50 MP (1.2 μm)

Manual Control	Deep (Pro Mode/API).	Limited for 3rd party

HFR.

Full.

C	C

While the Google Pixel series often provides a more "standard" implementation of the Camera2 API, the Samsung S25's Snapdragon 8 Elite provides superior raw processing power and memory bandwidth, which are critical for sustaining the 500 MB/s transfer required for 240 fps uncompressed capture. The primary advantage of the S25 for PIV researchers is the maturity of its "Pro" hardware paths, which, once unlocked via the correct Physical ID, offer the most stable high-

speed performance in the mobile market.

# Thermal and Power Considerations for High-Frame-Rate Capture

## Thermal Throttling in the S25

High-speed data acquisition is an energy-intensive process that generates substantial thermal loads. The Snapdragon 8 Elite's Oryon CPU cores and the Adreno 830 GPU operate at high clock speeds to manage the 240 fps intake, while the sensor itself generates heat during rapid readout.

The Vapor Chamber: The S25 includes an enlarged vapor chamber to disperse heat, but sustained 240 fps capture for more than 60 seconds will likely trigger thermal limits.

Performance Impact: Once thermal throttling occurs, the system's "Thermal Manager" may evict the camera client or force a frame rate reduction to 120 or 60 fps.

PIV Strategy: Acquisition should be performed in "Bursts." Using the circular buffer, the tool can remain in a low-power "idle" state, only engaging the high-speed pipeline and memory writes for the few seconds necessary to capture the relevant fluid dynamics event.

## Battery and Power Delivery

Capturing at 240 fps increases battery drain significantly. For field PIV, the S25's 4000–5000 mAh battery is sufficient for several dozen acquisition cycles. However, for laboratory use, the device should be powered via USB-C using a high-wattage (25W+) Power Delivery (PD) source. Note that charging during capture further increases thermal levels, so a "Bypass Charging" or "Pause USB Power Delivery" mode (found in Samsung's Game Booster settings) should be used to power the

device directly from the wall without charging the battery, minimizing heat.

# Integration of PIV Post-Processing Algorithms

Once the HighFPSRecorder_240fps has successfully acquired the raw Y-plane data into a circular buffer and saved it to UFS 4.0, the data is ready for analysis.

## Sub-Pixel Displacement and Temporal Delta

The fundamental PIV equation for velocity v is:

v =

Δx M ⋅ Δt

where Δx is the displacement in pixels, M is the magnification factor (pixels/meter), and Δt is the time between frames.

Magnification (M): Calibrated using a physical target of known size (e.g., a scale bar) at the same focal distance. The tool must lock focus to ensure M remains constant.

Time Interval (Δt): While the target is 240 fps (Δt = 4.167ms), actual inter-frame times can fluctuate slightly due to sensor jitter. The tool must use the nanosecond timestamps from the CaptureResult metadata for each frame pair to calculate the exact Δt for that specific

velocity vector.

## Accelerating Cross-Correlation on the Adreno 830

The S25's Adreno 830 GPU can be utilized to perform real-time or near-real-time PIV analysis using the Discrete Cross-Correlation (DCC) method.

- Fast Fourier Transforms (FFT): The correlation of two image windows is most efficiently computed in the frequency domain. The Snapdragon 8 Elite's GPU supports Vulkan and OpenCL, which can parallelize FFTs across thousands of cores.

- Vector Field Overlay: Once computed, the velocity vectors can be rendered as an overlay on the preview screen using OpenGL ES, giving researchers immediate feedback on the flow conditions.

# Final Implementation Strategy for the HighFPSRecorder_240fps

To enhance the developer's current repository and solve the 60 fps primary camera cap on the Samsung Galaxy S25, the following roadmap is recommended:

## Phase 1: Camera Enumeration and Discovery

Implement a discovery routine that queries all CameraCharacteristics for the device. 	Filter for physical IDs that support CONSTRAINED_HIGH_SPEED_VIDEO .

Log the StreamConfigurationMap for each ID to identify which sensor supports 1080p at 240

fps.

## Phase 2: NDK Data Pathway

Develop a C++ FrameBuffer class using the NDK to manage the circular buffer.

Implement a native correctStride function that handles the row-stride padding specific to the S25 sensors.

Use AImageReader_acquireNextImage in a high-priority native thread for the lowest possible latency.

## Phase 3: High-Speed Session Optimization

Configure the createConstrainedHighSpeedCaptureSession with only the ImageReader

surface and optionally a SurfaceTexture for preview.

Build a HighSpeedRequestList that sets all CONTROL_* keys to manual and locks the target FPS to ``.

## Phase 4: User Interface for Scientific Control

Add a "Scientific Mode" to the app that allows direct entry of SENSOR_EXPOSURE_TIME in microseconds and SENSOR_SENSITIVITY (ISO).

Implement a "Post-Trigger" capture logic: the app continuously fills the RAM circular buffer; when the user sees the flow event and hits "Capture," the app stops the camera and saves the last 4 seconds of data to storage.

# Conclusion and Strategic Outlook

The Samsung Galaxy S25, through its ISOCELL GN3/HP2 sensors and Snapdragon 8 Elite chipset, possesses the requisite hardware to serve as a high-fidelity data acquisition tool for

experimental fluid dynamics. The current limitation of 60 fps on the primary camera is not a hardware constraint but a software abstraction layer within the Samsung Camera HAL that prioritizes perceptual stability for third-party applications.

By transitioning from a logical camera interface to a physical ID targeting strategy, and by implementing an uncompressed Y-plane acquisition pipeline in the Android NDK, researchers can unlock the full 240 fps potential of the device. This approach, combined with nanosecond-accurate temporal metadata and manual hardware-level control over the sensor's exposure and sensitivity, allows for the creation of a Smartphone-Based PIV tool that rivals dedicated high-speed cameras in many laboratory and field scenarios. The future of mobile scientific imaging lies in this convergence of high-performance consumer hardware and expert-level software integration, transforming the S25 from a communication device into a powerful portable laboratory instrument.